**ACDT — Agentic Architecture Summary**

**Digital Twins**

- Vehicle Digital Twin (Eclipse Ditto) — represents the 2018 Toyota Camry, synced with live sensor data

- Mechanic's own separate Digital Twin (Eclipse Ditto) — represents the technician/service centre, independently provisioned

**Agentic Interactions**

1. **Driver ↔ Vehicle DT**

Driver dashboard chat lets the owner converse directly with the car's digital twin. Backed by a LangChain `AgentExecutor` that dynamically calls tools (`check_all_sensors`, `check_fault_scenarios`, `check_anomaly_score`, `get_maintenance_schedule`, etc.) rather than returning static/scripted responses — the agent reasons over live sensor data each time.

2. **Vehicle DT → Technician (emergency/maintenance reporting)**

When an emergency or maintenance condition is detected, the vehicle's twin pushes a packet to the mechanic's twin over Ditto's HTTP API (`push_to_mechanic()`), with local MongoDB fallback if the mechanic's twin is unreachable. The technician can then query for more detail through the console.

3. **Technician ↔ Technician's own DT**

The mechanic console interacts with a completely separate Ditto instance representing the service centre's own twin — not just a shared view of the vehicle's twin. This is twin-to-twin communication, not a single twin with two dashboards.

4. **Technician ↔ ATDT (diagnostic assistant)**

The technician can converse with ATDT to get symptom-based diagnoses and step-by-step repair guidance, also agent-driven (separate Diagnostic and Guidance LangChain agents), pulling from a Neo4j knowledge graph with `RepairProcedure` nodes.

**Models Driving the Agents**

All agents run on locally self-hosted open-weight models via Ollama — no cloud LLM API, no VLM (the system is text/sensor-data driven, not vision-based):

| Agent | Model |

|---|---|

| Safety monitor | gpt-oss:20b |

| Preventive maintenance | gpt-oss:120b |

| Predictive maintenance | gpt-oss:120b |

| ATDT Diagnostic | gpt-oss:120b |

| ATDT Guidance | gpt-oss:120b |

**Data Collection**

Live telemetry flows from an ESP32 + ELM327 OBD-II adapter over MQTT (HiveMQ Cloud) into the backend, with a Redis-based override letting a technician force the active data source (Simulator / Adapter / MQTT / Auto) for testing or live operation. Confirmed working end-to-end, including a live in-person demo where revving the engine visibly changed RPM on the mechanic console.